#include "define.h"

#include "clock.h"

BYTE data year;

BYTE data month;

BYTE data day;

BYTE data week;

BYTE data hour;

BYTE data minute;

BYTE data second;

BIT 	time_change;

BIT		clock_12h;

BIT 	clock_am_pm;// am 1 pm 0

BYTE data clock_state;

BYTE data timer0_hour;

BYTE data timer0_minute;

BYTE data timer1_hour;

BYTE data timer1_minute;

extern BYTE data	ms_counter;//mS counter

WORD	second_count1=0;

WORD	second_count2=0;

void clock_main(void);

void clock_write_reg(BYTE addr, BYTE value);//write hex to bcd

void clock_write_ram(BYTE addr, BYTE value);//write hex

BYTE clock_read_reg(BYTE addr);//read bcd and return hex

BYTE clock_read_ram(BYTE addr);// read hex and return hex

BYTE bcd2_hex(BYTE val);

void read_clock(void);

void clock_initial(void);

void clock_write_enable(BIT enable);

void data_write(BYTE val);

BYTE data_read(void);

void clock_main(void)

{

switch(clock_state)

{

case	CLOCK_INITIAL:

clock_initial();

clock_state = CLOCK_MONITOR;

break;

case	CLOCK_MONITOR:

if(second != clock_read_reg(SECOND_ADDR))// THE bit 7 is let clock run enable

{

read_clock();

ms_counter = 0;

time_change = 1;

}

break;

case	CLOCK_SET_YEAR:

year++;

if(year > 10)

year = 0;

clock_write_enable(1);

clock_write_reg(YEAR_ADDR, year);

clock_write_enable(0);

clock_state = CLOCK_MONITOR;

break;

case	CLOCK_SET_MONTH:

month++;

if(month > 12)

month = 1;

clock_write_enable(1);

clock_write_reg(MONTH_ADDR, month);

clock_write_enable(0);

clock_state = CLOCK_MONITOR;

break;

case	CLCOK_SET_DAY:

day ++;

if(day > day_number[month])

day = 1;

clock_write_enable(1);

clock_write_reg(DAY_ADDR, day);

clock_write_enable(0);

clock_state = CLOCK_MONITOR;

break;

case	CLCOK_SET_WEEK:

week++;

if(week > 6)

week = 0;

clock_write_enable(1);

clock_write_reg(WEEK_ADDR, week);

clock_write_enable(0);

clock_state = CLOCK_MONITOR;

break;

case	CLOCK_SET_HOUR:

hour++;

if(clock_12h)

{

if(clock_am_pm == 1)

{

if(hour > 12)

{

clock_am_pm = 0;

hour = 1;

}

}

else

{

if(hour > 12)

{

clock_12h = 0;

clock_am_pm = 0;

hour = 0;

}

}

}

else

{

if(hour == 24)

{

hour = 1;

clock_12h = 1;

clock_am_pm = 1;

}

}

clock_write_enable(1);

clock_write_reg(HOUR_ADDR, hour);

clock_write_enable(0);

clock_state = CLOCK_MONITOR;

break;

case	CLOCK_SET_MINUTE:

minute++;

if(minute == 60)

minute = 0;

clock_write_enable(1);

clock_write_reg(MINUTE_ADDR, minute);

clock_write_enable(0);

clock_state = CLOCK_MONITOR;

break;

case	CLOCK_SET_SECOND:

clock_write_enable(1);

clock_write_reg(SECOND_ADDR, 0x00);//let clock run

clock_write_enable(0);

clock_state = CLOCK_MONITOR;

break;

default :

clock_state = CLOCK_MONITOR;

break;

}

}

void clock_write_enable(BIT enable)

{

if(enable)

clock_write_reg(CONTROL_ADDR, 0X00);

else

clock_write_reg(CONTROL_ADDR, 0x80);

}

void clock_initial(void)

{

RTC_SCK = 0;

RTC_RST = 1;

RTC_SDA = 0;

if((clock_read_ram(RAM_0_ADDR) == 'W') && (clock_read_ram(RAM_1_ADDR) == 'K'))// had initialed

read_clock();

else

{

clock_12h = 0;

clock_write_enable(1);

clock_write_reg(SECOND_ADDR, second);//let clock run

clock_write_reg(MINUTE_ADDR, minute);

clock_write_reg(HOUR_ADDR, hour);//bit7 0->24hour mode

clock_write_reg(DAY_ADDR, day);

clock_write_reg(MONTH_ADDR, month);

clock_write_reg(WEEK_ADDR, week);

clock_write_reg(YEAR_ADDR, year);

clock_write_reg(CHARGE_ADDR, 0X01);

clock_write_ram(RAM_0_ADDR, 'W');

clock_write_ram(RAM_1_ADDR, 'K');

clock_write_enable(0);

}

}

/*****************************************************************/

/* THE RISE edge is vaild to write data                             */

/*****************************************************************/

void clock_write_reg(BYTE addr, BYTE value)

{

BYTE i;

i = value / 10;

i <<= 4;

i |= (value % 10);

value = i;

if(addr == SECOND_ADDR)

value &= 0x7f;

else if(addr == HOUR_ADDR)

{

if(clock_12h)

{

if(clock_am_pm)

value |= 0xa0;

else

{

value &= 0x1f;

value |= 0x80;

}

}

else

value &= 0x7f;

}

addr &= 0xfe;//set write state

RTC_SCK = 0;

RTC_RST = 0;

data_write(addr);

data_write(value);

RTC_RST = 1;

}

BYTE clock_read_reg(BYTE addr)

{

BYTE val = 0;

BYTE addr1 = addr;

addr |= 0x01;

RTC_SCK = 0;

RTC_RST = 0;

data_write(addr);

val = data_read();

RTC_RST = 1;

if(addr1 == SECOND_ADDR)

{

val &= 0x7f;

}

else if(addr1 == HOUR_ADDR)

{

if((val & 0x80) != 0)

{

clock_12h = 1;

if((val&0x20) != 0)

clock_am_pm = 1;

else

clock_am_pm = 0;

val &= 0x1f;

}

else

{

clock_12h = 1;

val &= 0x3f;

}

}

return(bcd2_hex(val));

}

void clock_write_ram(BYTE addr, BYTE value)

{

addr &= 0xfe;//set write state

RTC_SCK = 0;

RTC_RST = 0;

data_write(addr);

data_write(value);

RTC_RST = 1;

}

BYTE clock_read_ram(BYTE addr)

{

BYTE val = 0;

addr |= 0x01;

RTC_SCK = 0;

RTC_RST = 0;

data_write(addr);

val = data_read();

RTC_RST = 1;

return val;

}

BYTE bcd2_hex(BYTE val)

{

BYTE i;

i = val&0x0f;

val >>= 4;

val &= 0x0f;

val *= 10;

i += val;

return i;

}

void read_clock(void)

{

// include the am pm infomation

second	= clock_read_reg(SECOND_ADDR);

minute	= clock_read_reg(MINUTE_ADDR);

hour	= clock_read_reg(HOUR_ADDR);

week	= clock_read_reg(WEEK_ADDR);

day 	= clock_read_reg(DAY_ADDR);

month	= clock_read_reg(MONTH_ADDR);

year	= clock_read_reg(YEAR_ADDR);

}

void data_write(BYTE val)

{

BYTE i;

for(i = 0; i < 8; i++)

{

if((val & 0x01) != 0)

RTC_SDA = 1;

else

RTC_SDA = 0;

RTC_SCK = 0;

RTC_SCK = 1;

val >>= 1;// FOR DELAY AND SHIFT

}

}

BYTE data_read(void)

{

BYTE i,val;

RTC_SDA = 1;

for(i = 0; i < 8; i++)

{

RTC_SCK = 1;

RTC_SCK = 0;

val >>= 1;

if(RTC_SDA)

val |= 0x80;

}

return val;

}