HT1380实时时钟驱动程序

例程清单：
 
HT1380   EQU 0X06
COUNT0   EQU 0X10
TXBUF   EQU 0X11
RXBUF   EQU 0X12
DATABUF  EQU 0X13
ADDRPOINT  EQU 0X14
HT_SDA_IN  EQU 0B01001000
HD_SDA_OUT EQU 0B00001000
HT_SCL   EQU 0
HT_RST   EQU 1
Ht_SDA   EQU 2
;
TX_1380: MOV A,@HT_SDA_OUT
   IOW  HT1380
   BC  HT1380,HT_SCL
   MOV A,@8
   MOV COUNT0,A
TX_1380_LP: 
   BC  HT1380,HT_SDA
   JBC  TXBUF,0
   BS  HT1380,HT_SDA
   BS  HT1380,HT_SCL
   RRC  TXBUF
   BC  HT1380,HT_SCL
DJZ  COUNT0
JMP  TX_1380_LP
RET
;读HT1380子程序
READ_HT: 
   MOV TXBUF,A
   BC  HT1380,HT_SCL
   BS  HT1380,HT_RST
   MOV A,@HT_SDA_OUT
   IOW  HT1380
   MOV A,@8
   MOV COUNT0,A
TX_1_LP:
   BC  HT1380,HT_SDA
   JBC  TXBUF,0
   BS  HT1380,HT_SDA
   BS  HT1380,HT_SCL
   RRC  TXBUF
   BC  HT1380,HT_SCL
   DJZ  COUNT0
   JMP  TX_1_LP
   MOV A,@HT_SDA_IN
   IOW  HT1380
   MOV A,@8
   MOV COUNT0,A
RX_1380_LP:
   RRC  RXBUF
   BC  RXBUF,7
   JBC  HT1380,HT_SDA
   BS  RXBUF,7
   BS  HT1380,HT_SCL
   BC  HT1380,HT_SCL
   DJZ  COUNT0
   JMP  RX_1380_LP
   BC  HT1380,HT_RST
   RET
;写HT1380子程序
WRITE_HT:
   MOV ADDRPOINT,A
   BC  HT1380,HT_SCL
   BS  HT1380,HT_RST
   MOV A,@0B1000110
   MOV TXBUF,A
   CALL TX_1380
CLR  TXBUF
CALL TX_1380
BC  HT1380,HT_RST
NOP
BS  HT1380,HT_RST
MOV A,ADDRPOINT
   MOV TXBUF,A
   CALL TX_1380
MOV A,DATABUF
MOV TXBUF,A
CALL TX_1380
BC  HT1380,HT_RST
NOP
BS  HT1380,HT_RST
MOV A,@0B10001110
MOV TXBUF,A
CALL TX_1380
MOV A,@0B10000000
MOV TXBUF,A
CALL TX_1380
BC  HT1380,HT_RST
MOV A,HT_SDA_IN
IOW  HT1380
RET
END