Linux-2.6.25移植

4月17号linux-2.6.25稳定版发布，我们（伟研科技）http：//www.gzweiyan.com 将该内核移植到S3C2440平台上，下面是我们移植的第一步：

编译内核：

- 解压内核，当前目录为/new_disk/weiyan/

tar jxf /new_disk/weiyan/linux-2.6.25.tar.bz2

cd linux-2.6.25

- 修改体系架构并指定交叉编译器

vi Makefile ，将193行和194行的

#ARCH            ?= $(SUBARCH)

#CROSS_COMPILE   ?=

修改为

ARCH            ?= arm

CROSS_COMPILE   ?= arm-linux-	#最后有“—”，Makefile会调用arm-linux-gcc

- 载入默认配置

make s3c2410_defconfig

make menuconfig

按Esc键保存退出。

- 添加生成U-Boot镜像的工具

cp /new_disk/weiyan/mkimage ./

vi scripts/mkuboot.sh ，将

MKIMAGE=$(type -path "${CROSS_COMPILE}mkimage")

修改为MKIMAGE=$PWD/mkimage

vi arch/arm/mach-s3c2440/mach-smdk2440.c

- 添加nand划分信息

static struct mtd_partition wy_nand_part[] = {

[0] = {	//u-boot及内存存放的分区

.name   = "BOOT",

.size   = SZ_2M,

.offset = 0,

},

[1] = {	//文件系统存放的分区

.name   = "ROOTFS",

.offset = SZ_2M,

.size   = SZ_32M,

},

[2] = {	//剩余空间

.name   = "BACKUP",

.offset = SZ_32M + SZ_2M,

.size   = SZ_32M - SZ_2M,

},

};

static struct s3c2410_nand_set wy_nand_sets[] = {

[0] = {

.name        = "NAND",

.nr_chips     = 1,

.nr_partitions  = ARRAY_SIZE(wy_nand_part),

- 添加nand flash的读写匹配时间，各时间定义如图

static struct s3c2410_platform_nand wy_nand_info = {

.tacls          = 10,

.twrph0         = 25,

.twrph1         = 10,

.nr_sets        = ARRAY_SIZE(wy_nand_sets),

.sets           = wy_nand_sets,

};

.partitions     = wy_nand_part,

},

};

- 添加支持硬件校验：

make menuconfig

Device Drivers  --->

<*> Memory Technology Device (MTD) support  --->

<*>   NAND Device Support  --->

[*]     S3C2410 NAND Hardware ECC

- 编译并拷贝到tftp下载目录

make uImage && cp arch/arm/boot/uImage /new_disk/tftp

……

Image Name:   Linux-2.6.25

Created:      Mon Apr  7 13:50:19 2008

Image Type:   ARM Linux Kernel Image (uncompressed)

Data Size:    1591184 Bytes = 1553.89 kB = 1.52 MB

Load Address: 0x30008000

Entry Point:  0x30008000

Image arch/arm/boot/uImage is ready

启动新内核：

dm9000 i/o: 0x18000300, id: 0x90000a46

MAC: 00:0c:20:02:0a:5b

TFTP from server 192.168.1.5; our IP address is 192.168.1.6

Filename 'uImage'.

Load address: 0x31000000

Loading: #################################################################

#############################################

done

Bytes transferred = 1591268 (1847e4 hex)

## Booting image at 31000000 ...

Image Name:   Linux-2.6.25

Image Type:   ARM Linux Kernel Image (uncompressed)

Data Size:    1591204 Bytes =  1.5 MB

Load Address: 30008000

Entry Point:  30008000

Verifying Checksum ... OK

OK

Starting kernel ...

Uncompressing Linux....................................................................................................... done, booting the kernel.

Linux version 2.6.25 (root@ubuntu-server) (gcc version 3.4.1) #4 Mon Apr 21 04:28:37 CST 2008

CPU: ARM920T [41129200] revision 0 (ARMv4T), cr=c0007177

Machine: SMDK2440

Memory policy: ECC disabled, Data cache writeback

CPU S3C2440A (id 0x32440001)

S3C244X: core 406.425 MHz, memory 135.475 MHz, peripheral 67.737 MHz

S3C24XX Clocks, (c) 2004 Simtec Electronics

CLOCK: Slow mode (2.116 MHz), fast, MPLL on, UPLL on

CPU0: D VIVT write-back cache

CPU0: I cache: 16384 bytes, associativity 64, 32 byte lines, 8 sets

CPU0: D cache: 16384 bytes, associativity 64, 32 byte lines, 8 sets

Built 1 zonelists in Zone order, mobility grouping on.  Total pages: 16256

Kernel command line: root=/dev/mtdblock1 rootfstype=jffs2 console=ttySAC0 ip=192.168.1.6:192.168.1.5:192.168.1.5:255.255.255.0:WEIYAN:eth0:off

…….

S3C24XX NAND Driver, (c) 2004 Simtec Electronics

s3c2440-nand s3c2440-nand: Tacls=2, 14ns Twrph0=4 29ns, Twrph1=2 14ns

NAND device: Manufacturer ID: 0xec, Chip ID: 0x76 (Samsung NAND 64MiB 3,3V 8-bit)

Scanning device for bad blocks

Creating 3 MTD partitions on "NAND 64MiB 3,3V 8-bit":

0x00000000-0x00200000 : "BOOT"
0x00200000-0x02200000 : "ROOTFS"
0x02200000-0x04000000 : "BACKUP"

……

Empty flash at 0x0096200c ends at 0x00962200

Empty flash at 0x01470628 ends at 0x01470800

jffs2_scan_eraseblock(): Node at 0x01470dfc {0x1985, 0xe001, 0xe0021985) has invalid CRC 0x00000044 (calculated 0x515918d5)

VFS: Mounted root (jffs2 filesystem).

Freeing init memory: 140K

JFFS2 notice: (775) check_node_data: wrong data CRC in data node at 0x01470578: read 0x5cc80399, calculated 0x36a47240.

init started: BusyBox v1.9.1 (2008-04-19 19:49:32 CST)

starting pid 777, tty '': '/etc/init.d/rcS'

starting initial script, WEIYAN

Please press Enter to activate this console.

starting pid 795, tty '': '/bin/sh'

[root@WEIYAN /]$ ls

bin      etc      linuxrc  opt      sbin     tmp      var

dev      lib      mnt      proc     sys      usr

[root@WEIYAN /]$