/////////////////////////////////////////主文件//////////////////////////////////////////////////////////////

此程序包含两部分 主文件和子文件

/**

******************************************************************************

* @file    Project/Template/main.c

* @author  MCD Application Team

*本程序由 郭俊进行修改和全部的中文注释，便于大家学习与交流

*注意：1.在对引脚进行修改时必须在中断的子函数里也要修改引脚，否则程序不会执行

2.要将引脚boot0置于低电平，启动内部存储区的程序

******************************************************************************

**/

/***************************定义头文件***************************************/

#include "stm32f10x.h"

#include "stm32f10x_exti.h"

#include "misc.h"

ITStatus EXTIStatus;

/************************定义子函数*****************************/

void EXTI_Config(void);

void GPIO_Config(void);

void NVIC_Config(void);

void  Delay (u32 nCount)

{

for(; nCount != 0; nCount--);

}

int main(void)		 //主函数

{

SystemInit();	//系统初始化

GPIO_Config(); // 引脚定义

EXTI_Config(); //  中断配置

NVIC_Config(); //  潜入向量中断控制器

while (1)	 //死循环，LED1 2循环闪烁 ，LED3 4处于低电平，熄灭

{

GPIO_SetBits(GPIOC , GPIO_Pin_6);

GPIO_ResetBits(GPIOC , GPIO_Pin_7);

GPIO_ResetBits(GPIOD , GPIO_Pin_6);

GPIO_ResetBits(GPIOD , GPIO_Pin_13);

Delay(0xfffff);

Delay(0xfffff);	  //延时

Delay(0x5ffff);

GPIO_ResetBits(GPIOC , GPIO_Pin_6);

GPIO_SetBits(GPIOC , GPIO_Pin_7);

GPIO_ResetBits(GPIOD , GPIO_Pin_6);

GPIO_ResetBits(GPIOD , GPIO_Pin_13);

Delay(0xfffff);

Delay(0xfffff);

Delay(0x5ffff);

}

}

void GPIO_Config(void)		//	定义引脚

{

GPIO_InitTypeDef GPIO_InitStructure;

RCC_APB2PeriphClockCmd(RCC_APB2Periph_GPIOA | RCC_APB2Periph_GPIOB | RCC_APB2Periph_GPIOC |

RCC_APB2Periph_GPIOD | RCC_APB2Periph_GPIOE | RCC_APB2Periph_AFIO, ENABLE);   //设置时钟

/**

*	LED1 -> PC6	 ,	LED2 -> PC7 , LED3 -> PD6 , LED4 -> PD13

*  Key1 -> PE2  ,  Key2 -> PE3

*/

GPIO_InitStructure.GPIO_Pin = GPIO_Pin_6 |GPIO_Pin_7;	   //LED 1 2

GPIO_InitStructure.GPIO_Speed = GPIO_Speed_50MHz;

GPIO_InitStructure.GPIO_Mode = GPIO_Mode_Out_PP;

GPIO_Init(GPIOC, &GPIO_InitStructure);

GPIO_InitStructure.GPIO_Pin = GPIO_Pin_6 |GPIO_Pin_13 ; //LED3 4

GPIO_InitStructure.GPIO_Speed = GPIO_Speed_50MHz;

GPIO_InitStructure.GPIO_Mode = GPIO_Mode_Out_PP;

GPIO_Init(GPIOD, &GPIO_InitStructure);

GPIO_InitStructure.GPIO_Pin = GPIO_Pin_2 |GPIO_Pin_3;	 //中断引脚

GPIO_InitStructure.GPIO_Speed = GPIO_Speed_50MHz;

GPIO_InitStructure.GPIO_Mode = GPIO_Mode_IN_FLOATING;

GPIO_Init(GPIOE, &GPIO_InitStructure);

}

void EXTI_Config(void)

{

EXTI_InitTypeDef EXTI_InitStructure;

//EXTI_StructInit(&EXTI_InitStructure);

GPIO_EXTILineConfig(GPIO_PortSourceGPIOE, GPIO_PinSource2);  	//管脚选择

GPIO_EXTILineConfig(GPIO_PortSourceGPIOE, GPIO_PinSource3);

EXTI_ClearITPendingBit(EXTI_Line3);	 //清除中断线中断标志位 否则将进入死循环

EXTI_ClearITPendingBit(EXTI_Line2);

EXTI_InitStructure.EXTI_Mode = EXTI_Mode_Interrupt;	   //中断请求

EXTI_InitStructure.EXTI_Trigger = EXTI_Trigger_Rising;	 //上升沿触发

EXTI_InitStructure.EXTI_Line = EXTI_Line2 | EXTI_Line3;	  //使用中断线2 3

EXTI_InitStructure.EXTI_LineCmd = ENABLE;

EXTI_Init(&EXTI_InitStructure);

//EXTI_Cmd(EXTI , ENABLE);

}

void NVIC_Config(void)

{

NVIC_InitTypeDef NVIC_InitStructure;

NVIC_PriorityGroupConfig(NVIC_PriorityGroup_0);

NVIC_InitStructure.NVIC_IRQChannel = EXTI2_IRQChannel;	  //通道

NVIC_InitStructure.NVIC_IRQChannelPreemptionPriority = 0;

NVIC_InitStructure.NVIC_IRQChannelSubPriority = 0;

NVIC_InitStructure.NVIC_IRQChannelCmd = ENABLE;

NVIC_Init(&NVIC_InitStructure);

NVIC_PriorityGroupConfig(NVIC_PriorityGroup_1);

NVIC_InitStructure.NVIC_IRQChannel = EXTI3_IRQChannel;	  //通道

NVIC_InitStructure.NVIC_IRQChannelPreemptionPriority = 1;

NVIC_InitStructure.NVIC_IRQChannelSubPriority = 1;

NVIC_InitStructure.NVIC_IRQChannelCmd = ENABLE;

NVIC_Init(&NVIC_InitStructure);

}

/****************************程序结尾的语句 可直接使用**************************************/

#ifdef  USE_FULL_ASSERT

void assert_failed(uint8_t* file, uint32_t line)

{

while (1)

{  }

}

#endif

/******************* (C) COPYRIGHT 2009 STMicroelectronics *****END OF FILE****/

////////////////////////////////////////////////////////子文件/////////////////////////////////////////////////////////////////

/**

******************************************************************************

* @file    Project/Template/stm32f10x_it.c

* @author  MCD Application Team

* @version V3.0.0

* @date    04/06/2009

* @brief   Main Interrupt Service Routines.

*          This file provides template for all exceptions handler and

*          peripherals interrupt service routine.

******************************************************************************

* @copy

*

* THE PRESENT FIRMWARE WHICH IS FOR GUIDANCE ONLY AIMS AT PROVIDING CUSTOMERS

* WITH CODING INFORMATION REGARDING THEIR PRODUCTS IN ORDER FOR THEM TO SAVE

* TIME. AS A RESULT, STMICROELECTRONICS SHALL NOT BE HELD LIABLE FOR ANY

* DIRECT, INDIRECT OR CONSEQUENTIAL DAMAGES WITH RESPECT TO ANY CLAIMS ARISING

* FROM THE CONTENT OF SUCH FIRMWARE AND/OR THE USE MADE BY CUSTOMERS OF THE

* CODING INFORMATION CONTAINED HEREIN IN CONNECTION WITH THEIR PRODUCTS.

*

* <h2><center>&copy; COPYRIGHT 2009 STMicroelectronics</center></h2>

*/

/* Includes ------------------------------------------------------------------*/

#include "stm32f10x_it.h"

#include "stm32f10x_exti.h"

/** @addtogroup Template_Project

* @{

*/

/* Private typedef -----------------------------------------------------------*/

/* Private define ------------------------------------------------------------*/

/* Private macro -------------------------------------------------------------*/

/* Private variables ---------------------------------------------------------*/

/* Private function prototypes -----------------------------------------------*/

/* Private functions ---------------------------------------------------------*/

/**

* @brief  Delay program.

* @param  None

* @retval : None

*/

static void  Delay (u32 nCount)

{

for(; nCount != 0; nCount--);

}

/******************************************************************************/

/*            Cortex-M3 Processor Exceptions Handlers                         */

/******************************************************************************/

/**

* @brief  This function handles NMI exception.

* @param  None

* @retval : None

*/

void NMI_Handler(void)

{

}

/**

* @brief  This function handles Hard Fault exception.

* @param  None

* @retval : None

*/

void HardFault_Handler(void)

{

/* Go to infinite loop when Hard Fault exception occurs */

while (1)

{

}

}

/**

* @brief  This function handles Memory Manage exception.

* @param  None

* @retval : None

*/

void MemManage_Handler(void)

{

/* Go to infinite loop when Memory Manage exception occurs */

while (1)

{

}

}

/**

* @brief  This function handles Bus Fault exception.

* @param  None

* @retval : None

*/

void BusFault_Handler(void)

{

/* Go to infinite loop when Bus Fault exception occurs */

while (1)

{

}

}

/**

* @brief  This function handles Usage Fault exception.

* @param  None

* @retval : None

*/

void UsageFault_Handler(void)

{

/* Go to infinite loop when Usage Fault exception occurs */

while (1)

{

}

}

/**

* @brief  This function handles SVCall exception.

* @param  None

* @retval : None

*/

void SVC_Handler(void)

{

}

/**

* @brief  This function handles Debug Monitor exception.

* @param  None

* @retval : None

*/

void DebugMon_Handler(void)

{

}

/**

* @brief  This function handles PendSVC exception.

* @param  None

* @retval : None

*/

void PendSV_Handler(void)

{

}

/**

* @brief  This function handles SysTick Handler.

* @param  None

* @retval : None

*/

void SysTick_Handler(void)

{

}

/******************************************************************************/

/*                 STM32F10x Peripherals Interrupt Handlers                   */

/*  Add here the Interrupt Handler for the used peripheral(s) (PPP), for the  */

/*  available peripheral interrupt handler's name please refer to the startup */

/*  file (startup_stm32f10x_xx.s).                                            */

/******************************************************************************/

void EXTI2_IRQHandler(void)

{

if ( EXTI_GetITStatus(EXTI_Line2) != RESET ) {

EXTI_ClearITPendingBit(EXTI_Line2);

/*====LED34-ON=======*/

GPIO_ResetBits(GPIOC , GPIO_Pin_6);

GPIO_ResetBits(GPIOC , GPIO_Pin_7);

GPIO_ResetBits(GPIOD , GPIO_Pin_6);

GPIO_SetBits(GPIOD , GPIO_Pin_13);

Delay(0xfffff);

Delay(0xfffff);

Delay(0xfffff);

}

}

void EXTI3_IRQHandler(void)

{

if ( EXTI_GetITStatus(EXTI_Line3) != RESET ) {

EXTI_ClearITPendingBit(EXTI_Line3);	//清除中断标志

/*====LED34-ON=======*/

GPIO_ResetBits(GPIOC , GPIO_Pin_6);

GPIO_ResetBits(GPIOC , GPIO_Pin_7);

GPIO_SetBits(GPIOD , GPIO_Pin_6);

GPIO_ResetBits(GPIOD , GPIO_Pin_13);

Delay(0xfffff);

Delay(0xfffff);

Delay(0xfffff);

}

}

/******************* (C) COPYRIGHT 2009 STMicroelectronics *****END OF FILE****/