8.23多功能波形发生器VHDL程序与仿真

--文件名：mine4.vhd。

--功能：实现4种常见波形正弦、三角、锯齿、方波（A、B）的频率、幅度可控输出（方波

--A的占空比也是可控的），可以存储任意波形特征数据并能重现该波形，还可完成

--各种波形的线形叠加输出。

--说明： SSS（前三位）和SW信号控制4种常见波形种哪种波形输出。4种波形的频率、

--幅度(基准幅度A)的调节均是通过up、down、set按键和4个BCD码置入器以及一

--个置入档位控制信号（ss）完成的（AMP的调节范围是0~5V，调节量阶为1/51V）。

--其中方波的幅度还可通过u0、d0调节输出数据的归一化幅值（AMP0）进行进一步

--细调（调节量阶为1/（51*255）V）。方波A的占空比通过zu、zp按键调节（调节

--量阶1/64*T）。系统采用内部存储器——RAM实现任意输入波形的存储，程序只支

--持键盘式波形特征参数置入存储，posting 为进入任意波置入(set)、清除（clr）状态

--控制信号，SSS控制存储波形的输出。P180为预留端口，

--最后修改日期：2004.3.26。

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

use ieee.std_logic_unsigned.all;

entity mine4 is

port(clk : in std_logic;                                        --时钟信号输入

set, clr, up, down, zu, zd : in std_logic;              --各个波形特征的调节触发信号

posting : in std_logic;                                    --任意波键盘置入信号

u0,d0,sw : in std_logic;           --方波A、B的切换sw，和方波B的幅度调节按键

ss : in std_logic_vector( 3 downto 0 );                        --档位选择信号

sss : in std_logic_vector( 4 downto 0 );                       --波形选择信号

Data3, Data2, Data1,Data0 : in std_logic_vector(3 downto 0); --BCD码输入

p180 : out std_logic;                                      --预留接口

lcd : out std_logic_vector(7 downto 0);                       --显示输出

shift : out std_logic_vector(3 downto 0);                      --位码输出

dd, a : out std_logic_vector( 7 downto 0));                    --波形、幅度数据输出

end mine4;

architecture behav of mine4 is

subtype word is std_logic_vector( 7 downto 0 );

type  unit is array(63 downto 0) of word;

signal ram : unit;

signal qqq : integer range 0 to 250000000;

signal qq : integer range 0 to 78125000;

signal tmp : integer range 0 to 9999;

signal coun : integer range 0 to 78125000;

signal coun0 : integer range 0 to 250000000;

signal b : integer range 0 to 78125000;

signal c : integer range 0 to 500000000;

signal z, con : integer range 0 to 63;

signal f : std_logic_vector( 7 downto 0 );

signal amp, amp0, d : std_logic_vector(7 downto 0);

signal bcd0,bcd1,bcd2,bcd3 : integer range 0 to 9;

signal bcd01,bcd11,bcd21,bcd31 : integer range 0 to 9;

signal bcd00,bcd10,bcd20,bcd30 : integer range 0 to 9;

signal y : integer range 0 to 9;

signal addr : integer range 0 to 63;

begin

qq<=781250 when ss="1000" else

7812500 when ss="0100" else

78125000 when ss="0010" else

78125;

--qq信号对应SW=0时的档位选择信号SS，实现方波A和其他三种波形的频率预置

qqq<= 500000 when ss="1000" else

5000000 when ss="0100" else

50000000 when ss="0010" else

50000;

--qqq信号对应SW=1时的档位选择信号SS，实现方波B的频率预置

process(clk)

--此进程分别描述了各种波形的频率、幅度（方波A的占空比）调节以及各种波形的任意线

--形叠加等。

variable count4 : integer range 0 to 6250000;

variable count : integer range 0 to 78125000;

variable count3 : integer range 0 to 250000000;

variable count1 : integer range 0 to 12500000;

variable count0 : integer range 0 to 3249999;

variable ddd : std_logic_vector(9 downto 0);

variable dd0,dd1,dd2,dd3,dd4 : integer range 0 to 255;

variable adr : integer range 0 to 63;

begin

if rising_edge(clk) then

if posting='1' then

if count4=6249999 then count4:=0;

adr:=conv_integer(Data3)*10+conv_integer(Data2);--存储单位地址

if adr<64 then

if set='1' then ram(adr)<=conv_std_logic_vector((conv_integer(Data1)*10

+conv_integer(Data0))*2,8); --对置入的任意波形数据进行储存

elsif clr='1' then  adr:=0;                             --存储器所有单元清零

for i in 0 to 63 loop

ram(i)<=(others=>'0');

end loop;

end if;

end if;

else count4:=count4+1;

end if;

else

if set='1' then coun<=0; b<=0; coun0<=0;c<=0;z<=31;amp0<="01111111"; addr<=0;

tmp<=conv_integer(Data3)*1000+conv_integer(Data2)*100

+conv_integer(Data1)*10+conv_integer(Data0);      --频率数据

amp<="01111111";                                     --幅值

else

if tmp>0 then

if sw='0' then

if coun<qq then coun<=coun+tmp; b<=b+1;    --频率到采样点间隔脉冲数转换

else

if count=b then count:=1;

if f=63 then f<="00000000";

else f<=f+1;

end if;

if sss="00010" then                                      --方波A

if con<=z then  dd<=amp0; con<=con+1;

elsif con=63 then con<=0; dd<="00000000";

else con<=con+1; dd<="00000000";

end if;

elsif sss="10000" then dd<=d;                             --正弦波

elsif sss="00100" then dd<=f(5 downto 0)&"00";              --锯齿波

elsif sss="01000" then --三角波

if f>31 then dd<=("111111"-f(5 downto 0))&"00";

else dd<=f(5 downto 0)&"00";

end if;

elsif sss="00001" then                                   --任意波

if addr<63 then dd<=ram(addr); addr<=addr+1;

elsif addr=63 then dd<=ram(63); addr<=0;

end if;

else                                  --完成5种波形的线形叠加

if sss(1)='1' then

if con<=z then con<=con+1;

dd0:=conv_integer(amp0);      --方波波形数据dd0

else con<=con+1; dd0:=0;

end if;

end if;

if sss(4)='1' then dd1:=conv_integer(d);        --正弦波波形数据dd1

end if;

if sss(2)='1' then dd2:=conv_integer(f(5 downto 0)&"00");

--锯齿波波形数据dd2

end if;

if sss(3)='1' then

if f>31 then dd3:=conv_integer(("111111"-f(5 downto 0))&"00");

else dd3:=conv_integer(f(5 downto 0)&"00"); --三角波波形数据dd3

end if;

end if;

if sss(0)='1' then

if addr<63 then dd4:=conv_integer(ram(addr)); addr<=addr+1;

elsif addr=63 then dd4:=conv_integer(ram(63)); addr<=0;

end if;                                --任意波波形数据dd4

end if;

ddd:=conv_std_logic_vector((dd0+dd1+dd2+dd3+dd4),10);

--波形线形叠加输出

dd<=ddd(9 downto 2);

end if;

else count:=count+1;

end if;

end if;

else

if coun0<qqq then coun0<=coun0+tmp; c<=c+1;

else

if count3<=c/2 then count3:=count3+1; dd<=amp0;

elsif count3=c then count3:=1;dd<="00000000";

else count3:=count3+1; dd<="00000000";

end if;

end if;

end if;

end if;

if count1=12499999 then count1:=0;                       --调方波A的占空比

if zu='1' then

if z<63 then z<=z+1;

else z<=63;

end if;

elsif zd='1' then

if z>0 then z<=z-1;

else z<=0;

end if;

end if;

else count1:=count1+1;

end if;

if count0=3249999 then count0:=0;

--up、down对4种波形幅度调节，u0、d0进一步对方波进行幅度调节

if u0='1' then

if amp0<"11111111" then amp0<=amp0+1;

else amp0<="11111111";

end if;

elsif d0='1' then

if amp0>"00000000" then amp0<=amp0-1;

else amp0<="00000000";

end if;

elsif up='1' then

if amp<"11111111" then amp<=amp+1;

else amp<="11111111";

end if;

elsif down='1' then

if amp>"00000000" then amp<=amp-1;

else amp<="00000000";

end if;

end if;

else count0:=count0+1;

end if;

end if;

end if;

end if;

end process;

a<=amp;                        --将幅值输出。

cov_a:process(clk,amp,amp0)

--主要实现各波形幅度值到BCD码的转化，由于方波和其他三种波形的幅度调节方式、精

--度不同，因此对幅度的处理方式分两种：“sss="00010" or sw='1'”是判断输出波形是否为

--方波（A或B），bcd00,bcd10,bcd20,bcd30是本进程的输出。

variable count : integer range 0 to 50004225;

variable counter : integer range 0 to 500055;

variable count1,count0 : integer range 0 to 4999999;

begin

if rising_edge(clk) then

if sss="00010" or sw='1' then count0:=0;                                   --方波

if count1=4999999 then count1:=0; bcd0<=0; bcd1<=0; bcd2<=0; bcd3<=0;

count:=(conv_integer(amp))*(conv_integer(amp0))*769; --幅值运算

elsif count1=4999900 then count1:=count1+1;

bcd00<=bcd0; bcd10<=bcd1; bcd20<=bcd2; bcd30<=bcd3;  --数据输出

else count1:=count1+1;                         --二进制码到BCD码的数据转换

if count>9999999 then count:=count-10000000; bcd0<=bcd0+1;

elsif count>999999 then count:=count-1000000; bcd1<=bcd1+1;

elsif count>99999 then count:=count-100000; bcd2<=bcd2+1;

elsif count>9999 then count:=count-10000; bcd3<=bcd3+1;

else null;

end if;

end if;

else count1:=0;                                        --正弦波、三角波、锯齿波

if count0=4999999 then counter:=conv_integer(amp)*1961;

count0:=0; bcd01<=0; bcd11<=0; bcd21<=0; bcd31<=0;

elsif count0=4999000 then bcd00<=bcd01; bcd10<=bcd11; bcd20<=bcd21;

bcd30<=bcd31; count0:=count0+1;

else count0:=count0+1;

if counter>99999 then counter:=counter-100000; bcd01<=bcd01+1;

elsif counter>9999 then counter:=counter-10000; bcd11<=bcd11+1;

elsif counter>999 then counter:=counter-1000; bcd21<=bcd21+1;

elsif counter>99 then counter:=counter-100; bcd31<=bcd31+1;

else null;

end if;

end if;

end if;

end if;

end process;

process(clk)                             --输出波形幅度（峰-峰值）数据译码动态显示

variable count : integer range 0 to 499999;

begin

if rising_edge(clk) then

if count<=124999 then y<=bcd00; count:=count+1; shift<="0111"; lcd(0)<='0';

elsif count<=249999 then y<=bcd10; count:=count+1; shift<="1011";lcd(0)<='1';

elsif count<=374999 then y<=bcd20; count:=count+1; shift<="1101";lcd(0)<='1';

elsif count<499999 then y<=bcd30; count:=count+1; shift<="1110";lcd(0)<='1';

elsif count=499999 then y<=bcd30; count:=0; shift<="1110";lcd(0)<='1';

end if;

end if;

case y is                               --7段码译码

when 0 => lcd(7 downto 1)<="0000001";

when 1 => lcd(7 downto 1)<="1001111";

when 2 => lcd(7 downto 1)<="0010010";

when 3 => lcd(7 downto 1)<="0000110";

when 4 => lcd(7 downto 1)<="1001100";

when 5 => lcd(7 downto 1)<="0100100";

when 6 => lcd(7 downto 1)<="0100000";

when 7 => lcd(7 downto 1)<="0001111";

when 8 => lcd(7 downto 1)<="0000000";

when 9 => lcd(7 downto 1)<="0000100";

when others => lcd(7 downto 1)<="0000001";

end case;

end process;

ym:process(clk)                  --正弦波在一个周期内时域上的64个采样点的波形数据

begin

if rising_edge(clk) then

case f is

when "00000000"=> d<="11111111"  ; when "00000001"=> d<="11111110"  ;

when "00000010"=> d<="11111100"  ;when "00000011"=> d<="11111001"  ;

when "00000100"=> d<="11110101"  ; when "00000101"=> d<="11101111"  ;

when "00000110"=> d<="11101001"  ;when "00000111"=> d<="11100001"  ;

when "00001000"=> d<="11011001"  ; when "00001001"=> d<="11001111"  ;

when "00001010"=> d<="11000101"  ;when "00001011"=> d<="10111010"  ;

when "00001100"=> d<="10101110"  ; when "00001101"=> d<="10100010"  ;

when "00001110"=> d<="10010110"  ;when "00001111"=> d<="10001001"  ;

when "00010000"=> d<="01111100"  ; when "00010001"=> d<="01110000"  ;

when "00010010"=> d<="01100011"  ;when "00010011"=> d<="01010111"  ;

when "00010100"=> d<="01001011"  ; when "00010101"=> d<="01000000"  ;

when "00010110"=> d<="00110101"  ;when "00010111"=> d<="00101011"  ;

when "00011000"=> d<="00100010"  ; when "00011001"=> d<="00011010"  ;

when "00011010"=> d<="00010011"  ;when "00011011"=> d<="00001101"  ;

when "00011100"=> d<="00001000"  ; when "00011101"=> d<="00000100"  ;

when "00011110"=> d<="00000001"  ;when "00011111"=> d<="00000000"  ;

when "00100000"=> d<="00000000"  ; when "00100001"=> d<="00000001"  ;

when "00100010"=> d<="00000100"  ;when "00100011"=> d<="00001000"  ;

when "00100100"=> d<="00001101"  ; when "00100101"=> d<="00010011"  ;

when "00100110"=> d<="00011010"  ;when "00100111"=> d<="00100010"  ;

when "00101000"=> d<="00101011"  ; when "00101001"=> d<="00110101"  ;

when "00101010"=> d<="01000000"  ;when "00101011"=> d<="01001011"  ;

when "00101100"=> d<="01010111"  ; when "00101101"=> d<="01100011"  ;

when "00101110"=> d<="01110000"  ;when "00101111"=> d<="01111100"  ;

when "00110000"=> d<="10001001"  ; when "00110001"=> d<="10010110"  ;

when "00110010"=> d<="10100010"  ;when "00110011"=> d<="10101110"  ;

when "00110100"=> d<="10111010"  ; when "00110101"=> d<="11000101"  ;

when "00110110"=> d<="11001111"  ;when "00110111"=> d<="11011001"  ;

when "00111000"=> d<="11100001"  ; when "00111001"=> d<="11101001"  ;

when "00111010"=> d<="11101111"  ;when "00111011"=> d<="11110101"  ;

when "00111100"=> d<="11111001"  ; when "00111101"=> d<="11111100"  ;

when "00111110"=> d<="11111110"  ;when "00111111"=> d<="11111111"  ;

when others=> null;

end case;

end if;

end process;

p180<='1';

end behav;