8.20 电子时钟VHDL程序与仿真

1. 10进制计数器设计与仿真

（1）10进制计数器VHDL程序

--文件名：counter10.vhd。

--功能：10进制计数器，有进位C

--最后修改日期：2004.3.20

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity counter10 is

Port ( clk : in std_logic;

reset : in std_logic;

din : in std_logic_vector(3 downto 0);

dout : out std_logic_vector(3 downto 0);

c:out std_logic);

end counter10;

architecture Behavioral of counter10 is

signal count : std_logic_vector(3 downto 0);

begin

dout <= count;

process(clk,reset,din)

begin

if reset='0'then

count <= din ;

c<='0';

elsif rising_edge(clk) then

if count = "1001" then

count <= "0000";

c<='1';

else

count <= count+1;

c<='0';

end if;

end if;

end process;

end Behavioral;

- 10进制计数器仿真

2. 6进制计数器设计与仿真

（1）6进制计数器VHDL程序

--文件名：counter6.vhd。

--功能：6进制计数器，有进位C

--最后修改日期：2004.3.20

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity counter6 is

Port ( clk : in std_logic;

reset : in std_logic;

din : in std_logic_vector(2 downto 0);

dout : out std_logic_vector(2 downto 0);

c:out std_logic);

end counter6;

architecture Behavioral of counter6 is

signal count : std_logic_vector(2 downto 0);

begin

dout <= count;

process(clk,reset,din)

begin

if reset= '0' then

count <= din;

c<='0';

elsif rising_edge(clk) then

if count="101" then

count<="000";

c<='1';

else

count<=count+1;

c<='0';

end if;

end if;

end process;

end Behavioral;

（2）6进制计数器仿真

3. 6进制计数器设计与仿真

（1）24进制计数器VHDL程序

--文件名：counter24.vhd。

--功能：24进制计数器。

--最后修改日期：2004.3.20

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity counter24 is

Port ( clk : in std_logic;

reset : in std_logic;

din : in std_logic_vector(5 downto 0);

dout : out std_logic_vector(5 downto 0));

end counter24;

architecture Behavioral of counter24 is

signal count : std_logic_vector(5 downto 0);

begin

dout <= count;

process(clk,reset,din)

begin

if reset= '0' then

count <= din;

elsif rising_edge(clk) then

if count(3 downto 0)="1001" then

count(3 downto 0)<="0000";

count(5 downto 4)<=count(5 downto 4) +1;

else

count(3 downto 0)<=count(3 downto 0)+1;

end if;

if count="100011" then

count<="000000";

end if;

end if;

end process;

end Behavioral;

（2）24进制计数器仿真

4. 译码器设计

（1）译码器VHDL程序

--文件名：decoder.vhd。

--功能：将4bit二进制数译码，在LED上显示相应数字。

--最后修改日期：2004.3.20

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity decoder is

Port (din:in std_logic_vector(3 downto 0 );   --四位二进制码输入

dout:out std_logic_vector(6 downto 0) );  --输出LED七段码

end decoder;

architecture Behavioral of decoder is

begin

process(din)

begin

case din is

when "0000" => dout<="0000001";--0

when "0001" => dout<="1001111";--1

when "0010" => dout<="0010010";--2

when "0011" => dout<="0000110";--3

when "0100" => dout<="1001100"; --4

when "0101" => dout<="0100100";--5

when "0110" => dout<="0100000";--6

when "0111" => dout<="0001111";--7

when "1000" => dout<="0000000";--8

when "1001" => dout<="0000100";--9

when others => dout<="1111111";

end case;

end process;

end Behavioral;

5. 顶层设计与仿真

（1）顶层设计VHDL程序

--文件名：clock.vhd。

--功能：时钟的顶层设计。

--最后修改日期：2004.3.20

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity clock is

Port ( clk : in std_logic;                 --1Hz

reset : in std_logic;                --复位信号

dins : in std_logic_vector(6 downto 0);--秒钟预置

dinm : in std_logic_vector(6 downto 0);--分钟预置

dinh : in std_logic_vector(5 downto 0);--时钟预置

secondl: out std_logic_vector(6 downto 0);--秒钟低位输出

secondh: out std_logic_vector(6 downto 0); --秒钟高位输出

minutel: out std_logic_vector(6 downto 0); --分钟低位输出

minuteh: out std_logic_vector(6 downto 0); --分钟高位输出

hourl: out std_logic_vector(6 downto 0); --小时低位输出

hourh: out std_logic_vector(6 downto 0)); --小时高位输出

end clock;

architecture Behavioral of clock is

component counter10 is

Port ( clk : in std_logic;

reset : in std_logic;

din : in std_logic_vector(3 downto 0);

dout : out std_logic_vector(3 downto 0);

c:out std_logic);

end component;

component counter6 is

Port ( clk : in std_logic;

reset : in std_logic;

din : in std_logic_vector(2 downto 0);

dout : out std_logic_vector(2 downto 0);

c:out std_logic);

end component;

component counter24 is

Port ( clk : in std_logic;

reset : in std_logic;

din : in std_logic_vector(5 downto 0);

dout : out std_logic_vector(5 downto 0));

end component;

component decoder is

Port (din:in std_logic_vector(3 downto 0 );

dout:out std_logic_vector(6 downto 0));

end component;

signal c1,c2,c3,c4:std_logic;

signal doutsl,doutml:std_logic_vector(3 downto 0);

signal doutsh,doutmh:std_logic_vector(2 downto 0);

signal douth:std_logic_vector(5 downto 0);

signal rdoutsh,rdoutmh:std_logic_vector(3 downto 0);

signal rdouth:std_logic_vector(7 downto 0);

begin

rdoutsh <= '0'&doutsh;   --将秒钟高位数据变为4位，再进行译码

rdoutmh <= '0'&doutmh;  --将分钟高位数据变为4位，再进行译码

rdouth <="00"&douth;   --将时钟高位数据变为4位，再进行译码

u1: counter10 port map( clk=>clk,reset=>reset,

din=>dins(3 downto 0),

dout=>doutsl,

c=>c1);

u2: counter6 port map( clk=>c1,reset=>reset,

din=>dins(6 downto 4),

dout=>doutsh,

c=>c2);

u3: counter10 port map(	clk=>c2,reset=>reset,

din=>dinm(3 downto 0),

dout=>doutml,

c=>c3);

u4: counter6 port map( clk=>c3,reset=>reset,

din=>dinm(6 downto 4),

dout=>doutmh,

c=>c4);

u5: counter24 port map( clk=>c4,reset=>reset,

din=>dinh,

dout=>douth);

u6: decoder port map( din => doutsl,dout => secondl);	 --秒的低位

u7: decoder port map( din => rdoutsh,dout => secondh); --秒的高位

u8: decoder port map( din => doutml,dout => minutel);  --分的低位

u9: decoder port map( din => rdoutmh,dout => minuteh); --分的高位

u10: decoder port map( din => rdouth(3 downto 0),dout => hourh);--时的低位

u11: decoder port map( din => rdouth(7 downto 4),dout => hourl);--时的高位

end Behavioral;

（2）顶层设计仿真